تطبيق منهجية تقليل الفاقد في مصنع كابلات الطاقة الكهربائية: دراسة حالة  كابل الجهد المنخفض

محمد عمر العمودي اسم الطالب

إشراف

الدكتور/ رامي العمودي

المستلخص

على مدار العشرين عامًا الماضية،  أصبح تطبيق تطبيق منهجية  Lean   في جميع المجالات أحد أهم الاستراتيجيات المناسبة لكثير من الشركات الناجحة اليوم حيث أنه من خلال استخدام هذه الاستراتيجيات تتمكن الشركات من تحقيق نتائج مذهلة من حيث الجودة وزيادة الإنتاج وتقليل التكلفة. تبحث هذه الرسالة تطبيق المنهجية واستخدام بعض  اهم الادوات في  مصنع لإنتاج كابلات الجهد المنخفض بهدف تحفيز ورفع أداء الإنتاج وتقليل الوقت. من خلال هذه الرسالة تم اثبات أن طاقة المصنع الإجمالية بالإمكان زيادتها عن طريق تقليل 	وقت التوقف عن عمل المعدات مع زيادة الإنتاجية لها من خلال تنفيذ مباديء الصيانة الإنتاجية الإجمالية. أدت هذه الدراسة إلى تغيير رؤية العملاء لجودة المنتجات وقد تحقق ذلك من خلال تسليم المنتجات في الوقت المحدد وتقليل شكاوي العملاء.

Implementing Lean Methodology in a Power Cable Factory: A Case Study of Low Voltage Cable

Student Name: Mohammed Omar Alamoudi

Supervised by

Dr. Rami H. Alamoudi

Abstract

The thesis discusses the effects of implementing Lean methodology on low voltage cable factory where it enhances the performance of production and time. There are several different tools in the Lean methodology which were used in the research. These techniques include planned maintenance, autonomous maintenance, focused Improvement and overall equipment effectiveness, and daily performance Review. Pilot examples of how the use lean methods which are drum twister and extrusion machine. It is important to understand the current manufacturing process practiced in power cable factory. The practical effects and results of all these tools are highlighted such as the restoration of the drum twister machine transferred it to less breakdowns and speed increase where the production capacity increased by 35% per month, The extrusion machine speed increased by 80%, overall equipment effectiveness of drum twister increased by 26% and 20% for extrusion machine  and finally after applying Single-minute exchange of die approach 50% reduction of size change over from 180 minutes to 90 minutes achieved. Therefore, the companies often have to work with lean thinking. Finally, the thesis outlines the advantage and the benefits of incorporating lean methodology into a company’s projects and gives recommendation based on the observations such as top management support for the auditing system to follow-up standards deployment is very important to improve other machine across the factory.